DISCRETESEMICONDUCTORS

| DATA SHEET book,halfpage M3D067 BD825; BD829 NPN power transistors |  |  |  |
| --- | --- | --- | --- |

Product speciﬁcation Supersedes data of 1997 Jun 20

File under Discrete Semiconductors, SC04

1998 May 29

Philips Semiconductors	Product speciﬁcation

NPN power transistors	BD825; BD829

FEATURES

•	High current (max. 1 A)

•	Low voltage (max. 80 V).

| PINNING PIN | DESCRIPTION |
| --- | --- |
| 1 | emitter |
| 2 | collector, connected to metal part of mounting surface |
| 3 | base |

APPLICATIONS

•	General purpose

•	Driver stages in hi-fi amplifiers and television circuits.

DESCRIPTION

NPN power transistor in a TO-202; SOT128B plastic package. PNP complements: BD826 and BD830.

handbook,halfpage

2

3

1

1	2	3

MAM305

Fig.1 Simplified outline (TO-202; SOT128B) and symbol.

QUICK REFERENCE DATA

| SYMBOL | PARAMETER | CONDITIONS | MIN. | TYP. | MAX. | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VCBO | collector-base voltage BD825 BD829 | open emitter | − − | − − | 45 100 | V V |
| VCEO | collector-emitter voltage BD825 BD829 | open base | − − | − − | 45 80 | V V |
| ICM | peak collector current |  | − | − | 1.5 | A |
| Ptot | total power dissipation | Tamb≤25°C | − | − | 2 | W |
|  |  | Tmb≤50°C | − | − | 8 | W |
| hFE | DC current gain | IC = 150 mA; VCE = 2 V | 95 | − | 165 |  |
| fT | transition frequency | IC = 50 mA; VCE = 5 V; f = 100 MHz | − | 250 | − | MHz |

1998 May 29	2

Philips Semiconductors	Product speciﬁcation

NPN power transistors	BD825; BD829

LIMITING VALUES

In accordance with the Absolute Maximum Rating System (IEC 134).

| SYMBOL | PARAMETER | CONDITIONS | MIN. | MAX. | UNIT |
| --- | --- | --- | --- | --- | --- |
| VCBO | collector-base voltage BD825 BD829 | open emitter | − − | 45 100 | V V |
| VCEO | collector-emitter voltage BD825 BD829 | open base | − − | 45 80 | V V |
| VEBO | emitter-base voltage | open collector | − | 5 | V |
| IC | collector current (DC) |  | − | 1 | A |
| ICM | peak collector current |  | − | 1.5 | A |
| IBM | peak base current |  | − | 500 | mA |
| Ptot | total power dissipation | Tamb≤25°C | − | 2 | W |
|  |  | Tmb≤50°C | − | 8 | W |
| Tstg | storage temperature |  | −65 | +150 | °C |
| Tj | junction temperature |  | − | 150 | °C |
| Tamb | operating ambient temperature |  | −65 | +150 | °C |

THERMAL CHARACTERISTICS

| SYMBOL | PARAMETER | CONDITIONS | VALUE | UNIT |
| --- | --- | --- | --- | --- |
| Rth j-a | thermal resistance from junction to ambient | in free air | 62.5 | K/W |
| Rth j-mb | thermal resistance from junction to mounting base |  | 12.5 | K/W |

1998 May 29	3

Philips Semiconductors	Product speciﬁcation

NPN power transistors	BD825; BD829

CHARACTERISTICS

Tj = 25°C unless otherwise speciﬁed.

| SYMBOL | PARAMETER | CONDITIONS | MIN. | TYP. | MAX. | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| ICBO | collector cut-off current | IE = 0; VCB = 30 V | − | − | 100 | nA |
|  |  | IE = 0; VCB = 30 V; Tj = 125°C | − | − | 10 | μA |
| IEBO | emitter cut-off current | IC = 0; VEB = 5 V | − | − | 100 | nA |
| hFE | DC current gain | VCE = 2 V; see Fig.2 IC = 5 mA IC = 150 mA IC = 500 mA | 40 95 25 | − − − | − 165 − |  |
| VCEsat | collector-emitter saturation voltage | IC = 500 mA | − | − | 500 | mV |
| VBE | base-emitter voltage | IC = 500 mA; VCE = 2 V | − | − | 1 | V |
| fT | transition frequency | IC = 50 mA; VCE = 5 V; f = 100 MHz | − | 250 | − | MHz |

MBH729

160

handbook,fulpagewidth

hFE

VCE = 2 V

120

80

40

0

10−1

1	10	102	IC (mA)

103

Fig.2	DC current gain; typical values.

1998 May 29	4

Philips Semiconductors	Product speciﬁcation

NPN power transistors	BD825; BD829

PACKAGE OUTLINE

Plastic single-ended leaded (through hole) package; with cooling fin, mountable to heatsink,

1 mounting hole; 3 leads (in-line)	SOT128B

E1

c1

P

P1

HE

D

L1

L2

L

1

2

3

c

bp

w

M

Q

e1

A

e

E

0

5

10 mm

DIMENSIONS (mm are the original dimensions)

scale

L2(1)

UNIT

bp

D

E1

L1

L

max

c

c1

E

P

Q

w

e1

e

HE

P1

A

mm

0.8

0.6

0.65

0.5

0.56

0.46

8.6

8.4

10.1

9.9

10.4

10.0

2.54

13.3

12.2

5.08

24.2

23.8

2.4

2.0

3.8

3.6

3.9

3.7

1.7	0.25

1.5

2.5

4.6

4.4

Note

1. Plastic flash allowed within this zone

OUTLINE	REFERENCES

VERSION

EUROPEAN PROJECTION

ISSUE DATE

IEC

JEDEC

EIAJ

SOT128B

TO-202

97-02-28

1998 May 29	5

Philips Semiconductors	Product speciﬁcation

NPN power transistors	BD825; BD829

DEFINITIONS

| Data Sheet Status |  |
| --- | --- |
| Objective speciﬁcation | This data sheet contains target or goal speciﬁcations for product development. |
| Preliminary speciﬁcation | This data sheet contains preliminary data; supplementary data may be published later. |
| Product speciﬁcation | This data sheet contains ﬁnal product speciﬁcations. |
| Limiting values |  |
| Limiting values given are in accordance with the Absolute Maximum Rating System (IEC 134). Stress above one or more of the limiting values may cause permanent damage to the device. These are stress ratings only and operation of the device at these or at any other conditions above those given in the Characteristics sections of the speciﬁcation is not implied. Exposure to limiting values for extended periods may affect device reliability. |  |
| Application information |  |
| Where application information is given, it is advisory and does not form part of the speciﬁcation. |  |

LIFE SUPPORT APPLICATIONS

These products are not designed for use in life support appliances, devices, or systems where malfunction of these products can reasonably be expected to result in personal injury. Philips customers using or selling these products for use in such applications do so at their own risk and agree to fully indemnify Philips for any damages resulting from such improper use or sale.

1998 May 29	6

Philips Semiconductors	Product speciﬁcation

NPN power transistors	BD825; BD829

NOTES

1998 May 29	7

PhilipsSemiconductors–aworldwidecompany

Argentina: see South America

Australia: 34 Waterloo Road, NORTH RYDE, NSW 2113, Tel. +61 2 9805 4455, Fax. +61 2 9805 4466

Austria: Computerstr. 6, A-1101 WIEN, P.O. Box 213, Tel. +43 160 1010, Fax. +43 160 101 1210

Belarus: Hotel Minsk Business Center, Bld. 3, r. 1211, Volodarski Str. 6, 220050 MINSK, Tel. +375 172 200 733, Fax. +375 172 200 773

Belgium: see The Netherlands Brazil: see South America

Bulgaria: Philips Bulgaria Ltd., Energoproject, 15th floor, 51 James Bourchier Blvd., 1407 SOFIA,

Tel. +359 2 689 211, Fax. +359 2 689 102

Canada: PHILIPS SEMICONDUCTORS/COMPONENTS, Tel. +1 800 234 7381

China/Hong Kong: 501 Hong Kong Industrial Technology Centre, 72 Tat Chee Avenue, Kowloon Tong, HONG KONG,

Tel. +852 2319 7888, Fax. +852 2319 7700

Colombia: see South America Czech Republic: see Austria

Denmark: Prags Boulevard 80, PB 1919, DK-2300 COPENHAGEN S, Tel. +45 32 88 2636, Fax. +45 31 57 0044

Finland: Sinikalliontie 3, FIN-02630 ESPOO, Tel. +358 9 615800, Fax. +358 9 61580920

France: 51 Rue Carnot, BP317, 92156 SURESNES Cedex, Tel. +33 1 40 99 6161, Fax. +33 1 40 99 6427

Germany: Hammerbrookstraße 69, D-20097 HAMBURG, Tel. +49 40 23 53 60, Fax. +49 40 23 536 300

Greece: No. 15, 25th March Street, GR 17778 TAVROS/ATHENS, Tel. +30 1 4894 339/239, Fax. +30 1 4814 240

Hungary:	see Austria

India: Philips INDIA Ltd, Band Box Building, 2nd floor, 254-D, Dr. Annie Besant Road, Worli, MUMBAI 400 025, Tel. +91 22 493 8541, Fax. +91 22 493 0966

Indonesia:PT Philips Development Corporation, Semiconductors Division, Gedung Philips, Jl. Buncit Raya Kav.99-100, JAKARTA 12510,

Tel. +62 21 794 0040 ext. 2501, Fax. +62 21 794 0080

Ireland: Newstead, Clonskeagh, DUBLIN 14, Tel. +353 1 7640 000, Fax. +353 1 7640 200

Israel: RAPAC Electronics, 7 Kehilat Saloniki St, PO Box 18053, TEL AVIV 61180, Tel. +972 3 645 0444, Fax. +972 3 649 1007

Italy: PHILIPS SEMICONDUCTORS, Piazza IV Novembre 3, 20124 MILANO, Tel. +39 2 6752 2531, Fax. +39 2 6752 2557

Japan: Philips Bldg 13-37, Kohnan 2-chome, Minato-ku,

TOKYO 108-8507, Tel. +81 3 3740 5130, Fax. +81 3 3740 5077

Korea: Philips House, 260-199 Itaewon-dong, Yongsan-ku, SEOUL, Tel. +82 2 709 1412, Fax. +82 2 709 1415

Malaysia: No. 76 Jalan Universiti, 46200 PETALING JAYA, SELANGOR, Tel. +60 3 750 5214, Fax. +60 3 757 4880

Mexico: 5900 Gateway East, Suite 200, EL PASO, TEXAS 79905, Tel. +9-5 800 234 7381

Middle East: see Italy

Netherlands: Postbus 90050, 5600 PB EINDHOVEN, Bldg. VB, Tel. +31 40 27 82785, Fax. +31 40 27 88399

New Zealand: 2 Wagener Place, C.P.O. Box 1041, AUCKLAND, Tel. +64 9 849 4160, Fax. +64 9 849 7811

Norway: Box 1, Manglerud 0612, OSLO, Tel. +47 22 74 8000, Fax. +47 22 74 8341

Pakistan: see Singapore

Philippines: Philips Semiconductors Philippines Inc.,

106 Valero St. Salcedo Village, P.O. Box 2108 MCC, MAKATI, Metro MANILA, Tel. +63 2 816 6380, Fax. +63 2 817 3474

Poland: Ul. Lukiska 10, PL 04-123 WARSZAWA, Tel. +48 22 612 2831, Fax. +48 22 612 2327

Portugal: see Spain Romania: see Italy

Russia: Philips Russia, Ul. Usatcheva 35A, 119048 MOSCOW, Tel. +7 095 755 6918, Fax. +7 095 755 6919

Singapore: Lorong 1, Toa Payoh, SINGAPORE 319762, Tel. +65 350 2538, Fax. +65 251 6500

Slovakia: see Austria Slovenia: see Italy

South Africa: S.A. PHILIPS Pty Ltd., 195-215 Main Road Martindale, 2092 JOHANNESBURG, P.O. Box 7430 Johannesburg 2000,

Tel. +27 11 470 5911, Fax. +27 11 470 5494

South America: Al. Vicente Pinzon, 173, 6th floor, 04547-130 SÃO PAULO, SP, Brazil,

Tel. +55 11 821 2333, Fax. +55 11 821 2382

Spain: Balmes 22, 08007 BARCELONA,

Tel. +34 93 301 6312, Fax. +34 93 301 4107

Sweden: Kottbygatan 7, Akalla, S-16485 STOCKHOLM, Tel. +46 8 5985 2000, Fax. +46 8 5985 2745

Switzerland: Allmendstrasse 140, CH-8027 ZÜRICH, Tel. +41 1 488 2741 Fax. +41 1 488 3263

Taiwan: Philips Semiconductors, 6F, No. 96, Chien Kuo N. Rd., Sec. 1, TAIPEI, Taiwan Tel. +886 2 2134 2865, Fax. +886 2 2134 2874

Thailand: PHILIPS ELECTRONICS (THAILAND) Ltd.,

209/2 Sanpavuth-Bangna Road Prakanong, BANGKOK 10260, Tel. +66 2 745 4090, Fax. +66 2 398 0793

Turkey: Talatpasa Cad. No. 5, 80640 GÜLTEPE/ISTANBUL, Tel. +90 212 279 2770, Fax. +90 212 282 6707

Ukraine: PHILIPS UKRAINE, 4 Patrice Lumumba str., Building B, Floor 7, 252042 KIEV, Tel. +380 44 264 2776, Fax. +380 44 268 0461

United Kingdom: Philips Semiconductors Ltd., 276 Bath Road, Hayes, MIDDLESEX UB3 5BX, Tel. +44 181 730 5000, Fax. +44 181 754 8421

United States: 811 East Arques Avenue, SUNNYVALE, CA 94088-3409, Tel. +1 800 234 7381

Uruguay: see South America Vietnam: see Singapore

Yugoslavia: PHILIPS, Trg N. Pasica 5/v, 11000 BEOGRAD, Tel. +381 11 625 344, Fax.+381 11 635 777

For all other countries apply to: Philips Semiconductors,

International Marketing & Sales Communications, Building BE-p, P.O. Box 218, 5600 MD EINDHOVEN, The Netherlands, Fax. +31 40 27 24825

Internet: http://www.semiconductors.philips.com

©Philips Electronics N.V. 1998 SCA60 All rights are reserved. Reproduction in whole or in part is prohibited without the prior written consent of the copyright owner.

The information presented in this document does not form part of any quotation or contract, is believed to be accurate and reliable and may be changed without notice. No liability will be accepted by the publisher for any consequence of its use. Publication thereof does not convey nor imply any license under patent- or other industrial or intellectual property rights.

Printed in The Netherlands	135106/1200/03/pp8	Date of release: 1998 May 29	Document order number:	9397 750 03924